PART 2 – SECTION VII

EMPLOYER’S REQUIREMENTS

Introduction and Existing Condition

TABLE OF CONTENTS

ABBREVIATIONS	3

1	Introduction and Existing Situation	5

1.1	Introduction	5

1.2	Scope of works	5

1.3	Existing Conditions	7

1.3.1	Project area	7

1.3.2	Topography and geology	11

1.3.3	Seismicity	11

1.3.4	Climate and meteorology	12

1.3.5	Wastewater Collection System	12

1.3.6	Wastewater Treatment Plant	16

1.3.7	Receiving Body	20

1.4	Surveys	20

1.4.1	Topographical surveys	20

1.4.2	Geotechnical investigations	20

ABBREVIATIONS

| ATEX | Atmospheres Explosibles |
| --- | --- |
| BOD5 | Biochemical Oxygen Demand for 5 days |
| CA | Contracting Authority |
| CB | Circuit Breaker |
| COD | Chemical Oxygen Demand |
| DMS | Document Management System |
| DNP | Defects Notification Period |
| DS | Dry Solids |
| DSİ (SHW) | State Hydraulic Works |
| EC | Europen Commission |
| EIA | Environmental Impact Assessment |
| ESMP | Environmental and Social Management Plan |
| EMC | Electromagnetic Compatibility |
| EMP | Environmental Management Plan |
| EU | European Union |
| EUD | Delegation of the European Union to Turkiye |
| FDD | Final Design Deliverables |
| FDR | Final Design Report |
| FIDIC | International Federation of Consulting Engineers |
| GRP | Glass Reinforced Plastic |
| HDPE | High Density Polyethylene |
| ISO | International Organization for Standardization |
| MLSS | Mixed Liquor Suspended Solids |
| MoM | Minutes of Meeting |
| OPEX | One Year Operation Cost |
| ORP | Oxygen Reduction Potential |
| OS | Operating Structure |
| PDD | Preliminary Design Deliverables |
| PDR | Preliminary Design Report |
| PE | Polyelectrolyte |
| PID | Piping and Instrumentation Diagram |
| PIU | Project Implementation Unit |
| PLC | Programmable Logic Controller |
| PP | Polypropylene |
| PPRC | Polypropylene Random Copolymer |
| PSC | Project Steering Committee |
| PVC | Polyvinyl Chloride |
| QA | Quality Assurance |
| RAS | Return Activated Sludge |
| SCADA | Supervisory Control and Data Acquisition |
| SEP | Stakeholder Engagement Plan |
| SOR | Standard Oxygen Requirements |
| SOTE | Standard Oxygen Transfer Efficiency |
| SS | Suspended Solids |
| SSOTE | Standard Specific Oxygen Transfer Efficiency |
| T.R. | Republic of Turkiye |
| TEDAŞ | Turkish Electricity Distribution Corporation |
| TN | Total Nitrogen |
| TP | Total Phosphorus |
| TSS | Total Suspended Solids |
| UV | Ultraviolet |
| WB | World Bank |
| WAS | Waste Activated Sludge |
| WWTP | Wastewater Treatment Plant |

# Introduction and Existing Situation

## Introduction

The overall objective of the project is to increase the capacity of the existing Erbaa wastewater treatment plant to fulfill the requirements of wastewater management and to serve the increasing population. “Erbaa Existing Wastewater Treatment Plant Capacity Expansion Project” will be financed by TEFWER and will be evaluated under ‘TEFWER Component 1, Subcomponent 1.c - Restoration and resilience of water supply and wastewater services.

The specific aim of the project is to increase existing Erbaa wastewater treatment plant for municipal wastewater of Erbaa Municipality, which will meet both Turkish and EU standards in order to reduce the pollution loads to Kelkit Creek and to minimize the environmental pollution, prevent risk of ground water contamination.

## Scope of works

The works covered under this contract include the construction of Erbaa Wastewater Treatment Plant Capacity Expansion within the boundaries of allocated WWTP area.

The existing Erbaa wastewater treatment plant is the activated sludge plant with nutrient removal and discharged to Kelkit Creek with the existing discharge pipeline. Capacity increase will be the activated sludge plant with nutrient removal to meet discharge standards given under Document No 2 (2.5.1 Effluent Standards) and discharged to Kelkit Creek with the existing discharge pipeline.

The scope of the Contract includes the engineering design, supply, installation, and construction of all civil, mechanical, and electrical components required for the expansion of the wastewater treatment plant. It also includes testing, site acceptance, operational training for the end recipient's personnel, and delivery of a fully functional facility to the Municipality within the designated WWTP site boundaries.

The works will be implemented under the General Conditions of Contract as set out in the World Bank’s Standard Procurement Document for Plant: Design, Supply and Installation (D-S-I), as applicable to this project. Currently, Erbaa Wastewater Treatment Plant is in operation with a capacity of 11,640 m3/day with wastewater line consisting of inlet structure, anaerobic bio phosphorus tank, aeration tank, final settling tank and sludge line consisting of recirculation, excess sludge pump station and sludge dewatering building. The works to be carried out under the scope of work are explained below and in these sections of the Employers' Requirements. The existing 2 Aeration Tanks, 2 Final Settling Tanks and Sludge Dewatering Building shall be protected in the plant to be completed within the scope of work to be commissioned by the Contractor. Other structures listed below shall be demolished within the scope of the works. All demolishing works shall be carried out based on the continuation of the operation of the existing plant. Therefore, construction works shall be planned and carried out in stages, ensuring that any unavoidable interruptions in operation are minimized in both duration and impact. Bidders shall propose a detailed Testing and Implementation Phasing Methodology to this end with their proposal. This methodology shall be presented in the Work Plan and shall describe transitional phases, mitigation measures, and how operational continuity will be maintained. The Employers Requirements of the Tender Documents, which sets out the technical requirements and specifications, covers the following scope of works:

- Complete engineering designs for Stage 3 requirement including topographical and geological surveys,

- Demolishing of the following existing structures:

  - All inlet structures including in the existing screen building, grit removal structure and inlet pumping station (H-1 and H-2).

  - Final sedimentation distribution chamber (D3)

  - Discharge manhole (R-2)

  - Existing Return Activated Sludge Pumping Station (D-4).

  - Existing anaerobic biophosphorus tank (H-3 and D-1)

  - Blower Building

  - Transformer Building

- - Administration Building

- Supply and installation of sludge dewatering equipment into the existing dewatering building including polyelectrolyte preparation and all required auxiliary equipment and instruments. The renovations required for the installation of new equipment in the existing building shall be made and completed within the scope of the work.

- Renovation of the existing aeration tanks’ mixers, diffusers, jib cranes

- Renovation of the existing final sedimentation tanks’ scrapers and bridges

- Renovation of all weir plates and scum baffles in order to secure (and level) them in the existing aeration tanks and the existing final sedimentation tanks.

- Undertaking all necessary activities that secures operation of the existing WWTP facilities and agreed effluent standard on the outlet during the Contract period. Construction of a new administration building.

- Construction of new aeration tank and final sedimentation tank with the same dimensions and structural features as the existing tanks.

- Construction of the following units for the Stage capacity of WWTP as given in Document No 2 (2.2 Hydraulic Loads and Hydraulic Profile) and supply installation of all mechanical and electrical equipment and instrumentation. The isolation valves, penstocks, dismantling joints and valves including all required instruments shall be supplied and installed together with the units.:

  - Construction of new discharge pipeline with 800 mm diameter

  - New Inlet Structures (coarse screens, perforated fine screens, inlet pumping station, aerated grit and grease removal unit, and inlet flow measurement unit) .

  - Construction of new bio phosphorous tanks Construction of new distribution chambers (bioP tank distribution chamber, aeration tank distribution chamber, final sedimentation distribution chamber)

  - Construction of new Return Activated Sludge and Excess Sludge Pumping Station .

  - Construction of a solar drying unit

  - Construction of Blower building for aeration tanks,

  - Construction of Blower building for grit and grease removal units,

  - Construction of inlet, bypass, retun sludge and outlet flow meter chambers.

  - Construction of Odour Removal Unit,

  - Construction of Transformer Building

- All connections to the existing WWTP.

- Installation of electrical and mechanical equipment and all auxiliary facilities for the operation of the plant.

- Install an online monitoring system (CCTV system) for the effluent according to Turkish legislation.

- Install a CCTV for security and monitoring the site.

- Supply and installation of SCADA control system for whole WWTP;

- Supply and installation of instrumentation and associated PLCs.

- Supply and installation of power supply and electrical systems.

- Installation of new site lighting to cover the new treatment stage

- Testing, initial functional testing, performance demonstration period and commissioning and testing of the plant for 90 days

- Supply of Spare Parts needed for all facilities operation and maintenance during 2 years.

- Preparation of as-built documentation and operation and maintenance manuals;

- Installation of safety sign in Turkish language throughout the treatment plant.

- Supply of Operations and Maintenance Manuals in the Turkish and English languages.

- Training of staff in the operations and maintenance of the works/facilities.

- Management / supervision of operation & maintenance of the wastewater treatment plant during Defects Notification Period (DNP).

The implementation design of the works will be subject to the Project Manager’s approval.

Any drawings included in the contract are indicative in the implied sizing or location of units. The Contractor shall be liable for the implementation design of the works.

The Contractor shall supervise the Employer’s staff in their operation and maintenance of the wastewater treatment plant for the duration of the Commissioning Period. The Erbaa Municipality shall recruit operation staff to be trained. The practical training of staff (management, operation, maintenance and service) by the main contractor will take place continuously both during the construction and installation of equipment as well as during the site acceptance and testing and plant operation.

## Existing Conditions

### Project area

Erbaa, a district of Tokat province in the Central Black Sea region, is administratively composed of a district municipality (Erbaa Municipality), three town municipalities (Evyaba, Alacabal, and Karşıyaka), 74 villages, and 97 hamlets. Erbaa district center consists of 25 neighborhoods and the towns consist of 19 neighborhoods. There is a total of 118 village and neighborhood mukhtar offices in the district. The central coordinates of Erbaa district, located where Kelkit and Tozanlı streams merge and take the name of Yeşilırmak (Iris), are 40° 40' 21.8136'' North and 36° 34' 16.4892'' It is east and the distance to Tokat city center is approximately 79 kilometers. It is surrounded by Ayvacık district of Samsun and Akkuş district of Ordu province from the north, Taşova district of Amasya from the west, Niksar district of Tokat from the east, Tokat center from the south and Turhal district from the southwest.

The district has a surface area of 1111 square kilometers, which includes a plain along the Kelkit River, especially the Tozanlı Stream, and the surrounding mountainous areas.

The land slope is low within the zoning plan. The slope increases towards the south. Low-sloping lands in the north are generally agricultural lands.

The location of Erbaa district is given in Figure 1 below.

Figure 1  Location of the project area in Türkiye

Erbaa Wastewater Treatment Plant, which is the project area, is located in Erek Neighborhood, Çorak Mevkii, Block 1367, Parcel 1. The location of the plant is given below.

Figure 2  Erbaa WWTP

Source: Google Earth

Images of Erbaa Wastewater Treatment Plant are given below.

Figure 3  Erbaa WWTP

Source: https://erbaa.bel.tr/Site/foto/atik-su-aritma-tesisi

### Topography and geology

Erbaa is located in the Central Black Sea region of Turkiye. The district is surrounded by Çarşamba district of Samsun and Akkuş district of Ordu to the north, Taşova district of Amasya to the west, Niksar district of Tokat to the east, Tokat city center to the south, and Turhal district to the southwest.

Erbaa has an average elevation of 239 meters, making it the lowest-altitude district in Tokat province. The district's strategic location, with the Erbaa Plain stretching 32 km in length and 10 km in width, makes it highly conducive to agricultural activities. The district covers an area of 1,111 square kilometers, consisting of plains, particularly along the Kelkit River and partially along the Tozanlı Stream, as well as surrounding mountainous areas. The Karince Mountain in the north and the Sakarat and Boğali Mountains in the south are part of the Northern Anatolian Mountain range. With an average altitude of 1,000–1,250 meters, these mountains are relative-ly low compared to the general elevation of the Black Sea region, allowing for significant mari-time climate influence.

### Seismicity

The project area is within the limits of "0.748 g hazard" in the Earthquake Hazard Map of Turkey (10% probability of exceedance in 50 years - recurrence interval 475 years) (Turkey Earthquake Hazard Map). The provisions of Turkey Building Earthquake Regulations (TBDY) and Disaster Regulations for Infrastructure should be complied with.

The earthquake hazard map in Gazi Osman Paşa Neighborhood, which is close to the WWTP location of Erbaa District, is given in Figure 4.

Figure 4 Erbaa Earthquake Hazard Map

Source: https://tdth.afad.gov.tr/TDTH/main.xhtml

The project site, designated for the construction of a wastewater treatment plant (WWTP), is located within the “DD-2 Earthquake Ground Motion” zone as defined by the Turkish Building Earthquake Code (TBEC 2018). DD-2 corresponds to the design basis earthquake, which has a 10% probability of exceedance in 50 years and an approximate return period of 475 years. As the facility constitutes critical infrastructure, all structural and seismic design works related to the construction of the WWTP shall comply with the applicable requirements of TBEC 2018 for DD-2 level ground motions..

### Climate and meteorology

Two types of climates prevail in Erbaa. The northern parts of the district experience a Black Sea climate, while the southern areas are influenced by the continental climate of Central Anatolia. The North Anatolian Mountains act as a natural barrier, limiting the movement of the Black Sea climate towards the interior. Precipitation is higher in the northern parts, while the southern regions receive less rainfall.

Table 1. Temperatures and Precipitation Amounts in Erbaa

| Months | Average Temperature (°C) | Average Highest Temperature (°C) | Average Lowest Temperature (°C) | Average Number of Days with Precipitation | Average Precipitation (kg/m²) |
| --- | --- | --- | --- | --- | --- |
| January | 2.1 | 14.8 | -6.5 | 12.3 | 42.1 |
| February | 3.6 | 17.2 | -5.3 | 11.5 | 39.6 |
| March | 7.8 | 22.1 | -2.2 | 12.8 | 48.7 |
| April | 13.2 | 28.3 | 2.5 | 14.2 | 59.4 |
| May | 17.6 | 32.8 | 7.1 | 14.8 | 74.5 |
| June | 22.1 | 37.5 | 12.3 | 10.9 | 69.3 |
| July | 25.3 | 41.2 | 15.7 | 6.2 | 37.8 |
| August | 24.9 | 40.5 | 15.3 | 5.8 | 34.2 |
| September | 20.3 | 36.7 | 9.6 | 6.3 | 32.6 |
| October | 14.6 | 30.2 | 4.2 | 9.7 | 40.1 |
| November | 8.1 | 24.1 | -0.3 | 10.5 | 36.2 |
| December | 4.0 | 18.9 | -4.9 | 12.1 | 39.5 |

Source: www.mgm.gov.tr

The average annual temperature in Erbaa, based on data from 1964 to 1995, is 14°C, with the highest temperatures recorded in July and August and the lowest in January. Summers are relatively hot, while winters are milder compared to the city's immediate surroundings. The average summer temperature is 22.7°C, whereas the average winter temperature is 4.9°C.

The region receives an average annual rainfall of 440.4 mm. Summers are relatively dry, with an average precipitation of 68.2 mm, whereas winters are wetter, receiving around 125.1 mm of rainfall. The highest rainfall occurs in spring (32.6% of the annual total), while the lowest is in summer (15.5%).

The predominant wind direction in Erbaa is from the south-southwest, with an average speed of 1.5 m/sec. The highest recorded wind speed is 26.8 m/sec, coming from the south. Winds in the project area generally fall within the "light wind" category, with speeds around 6 m/sec over the long term.

### Wastewater Collection System

Sewerage and stormwater projects of Erbaa District were completed in 2015. Erbaa Municipality has completed 90% of the investments envisaged in these projects as of 2023 and the remaining parts are being completed one by one. The total number of waste water lines consisting of Ø200 mm, Ø300 mm, Ø400 mm, Ø500 mm, Ø600 and Ø800 mm diameter concrete pipes is 160 km.

Although the sewerage infrastructure of Evyaba region was designed in the sewerage project, the connection to the main sewerage channel in the zoning area was not considered, therefore, although the wastewater to be generated in Evyaba region was collected, it was planned to be disposed of with septic tanks within the region.

In the Sewerage Final Project commissioned by Iller Bank, the expected wastewater peak flows for 2014, 2029 and 2049 are 150 l/s, 181 l/s and 239 l/s, respectively, and include only the wastewater in the zoning areas with sewerage infrastructure. Wastewater in Evyaba and Alacabal areas (F1, F2), which are outside the zoning areas, are disposed of in local septic tanks without being conveyed to the treatment plant. Today, wastewater in sewerage channels that do not end with a treatment plant may need to be transferred to treatment plants with the help of vacuum trucks from time to time due to the filling of cesspools.

Information on sewerage and stormwater infrastructure is given in the table below.

Table 1 Information on Erbaa District Sewerage and Rainwater infrastructure

| Project Approval Date | 24.02.2015 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Project Area Population in the Last Census Year | 2013 Address Based Population Census 60,742 people |  |  |  |  |  |  |
| Project Target Year and Population | 2049 | 100,000 |  |  |  |  |  |
| Special Flow rates | Qend1(Slaughterhouse) |  |  | : 1.00 lt/sec |  |  |  |
|  | Qend2(M.Y. School) |  |  | : 2.00 lt/sec |  |  |  |
|  | Qend3(University) |  |  | : 2.00 lt/sec |  |  |  |
|  | Qend4(Hospital) |  |  | : 2.00 lt/sec |  |  |  |
|  | Qend (Total) |  |  | : 7.00 lt/sec |  |  |  |
| Project Flow Rate | No 1 Region |  | : 228.00+7.00=235.00 lt/sec |  |  |  |  |
|  | No 2F Region: 2.00 lt/sec |  |  |  |  | (Evyapa Neighb. Septic Tank Region) |  |
|  | No 3F Region: 2.00 lt/sec |  |  |  |  | (Alacabal Neighb. Septic Tank Region) |  |
|  | TOTAL |  | : |  |  |  |  |
| Development Plan Approval Year | 2014 Approved Final Development Plan, 82 1/1000 |  |  |  |  |  |  |
| Zoning Plan Area | 1950 Ha (Gross) |  |  |  |  |  |  |
| Density Zones | No 1 Region: 770 Ha Net, 98500-person, 128 person/Ha |  |  |  |  |  |  |
|  | No 2F Region: 15 Ha Net,750-person, 50 person/Ha (Evyapa Neighb.) |  |  |  |  |  |  |
|  | No 3F Region: 15 Ha Net, 750-person, 50 person/Ha (Alacabal Neighb.) |  |  |  |  |  |  |
| Sewerage Network Line Lengths by Diameter |  |  |  |  |  |  |  |
| Diameter | Stage 1 (m.) |  |  |  | Stage 2 (m.) |  | Existing (m.) |
| Ø200 | 170,221 |  |  |  | 91,855 |  | 5,230 |
| Ø300 | 12,153 |  |  |  |  |  |  |
| Ø400 | 5,317 |  |  |  |  |  | 1,172 |
| Ø500 | 4,184 |  |  |  |  |  |  |
| Ø600 | 3,917 |  |  |  |  |  |  |
| Ø800 | 2,201 |  |  |  |  |  |  |
| TOTAL |  |  | 197,993 |  |  | 91,855 | 6,402 |
| Collector Length and Diameter | Ø 800 Concrete Pipe L=1,901 m. |  |  |  |  |  |  |
| Length and diameter of discharge line | Ø 800 Concrete Pipe L=300 m. |  |  |  |  |  |  |
| Discharge point (receiving environment) | Zone No. 1: Wastewater treated in the existing treatment plant located in the north of the zoning plan will be discharged to Kelkit Stream. |  |  |  |  |  |  |
| Proposed treatment type and septic tank structure | Region No. 1: Activated Sludge with long aeration (existing) |  |  |  |  |  |  |
|  | Area No. 2F; Septic Structure (1st St. for 500 people) |  |  |  |  |  |  |
|  | Area No. 3F; Septic Tank Structure (1st St. for 500 persons) |  |  |  |  |  |  |
| Treatment Plant Location and Area required for the Plant | Region No. 1 Existing Treatment Plant; On the northern border of the zoning plan, on the left bank of the Kelkit Stream, on an area of approximately 1 Ha. |  |  |  |  |  |  |
| Existing Network Information | In general, there are network lines of various diameters that are not in good condition. Some of the existing lines were preserved in the project. |  |  |  |  |  |  |
| Stormwater Network | Stormwater project has been prepared by taking into consideration the requests of the Municipality. In addition, the existing sewer lines that have been cancelled can be used for the removal of rainwater by making the necessary arrangements. |  |  |  |  |  |  |
| Stormwater by Diameter | Length of Collector Lines |  |  |  |  |  |  |
|  | Stage 1 (m.) |  |  | Stage 2 (m.) |  |  | Existing (m.) |
| Ø400 | 1,490 |  |  |  |  |  |  |
| Ø500 | 2,343 |  |  |  |  |  |  |
| Ø600 | 1,036 |  |  |  |  |  |  |
| Ø800 | 2,243 |  |  |  |  |  |  |
| Ø1000 | 360 |  |  |  |  |  |  |
| Ø1400 | 324 |  |  |  |  |  |  |
| Ø1600 | 1,020 |  |  |  |  |  |  |
| 2,0m.x1,5m. | 264 |  |  |  |  |  |  |
| 3,0m. x2,0m. | 56 |  |  |  |  |  |  |
| TOTAL | 9,136 |  |  |  |  |  |  |

Figure 5  Erbaa District Sewerage and Stormwater Infrastructure System

Source: https://www.Erbaa Belediyesi.gov.tr

### Wastewater Treatment Plant

The Erbaa Wastewater Treatment Plant was one of the grant projects within the scope of the "Development Program for SDG Level 2 Regions" financed by the EU and the Republic of Turkiye in 2008. With the commissioning of the WWTP, the pollution loads in Kelkit Stream and Yeşilırmak River, where domestic and industrial wastewater is discharged untreated, have been reduced and the threats to nature and human health have been eliminated. Erbaa Wastewater Treatment Plant continues to serve Erbaa.

The capacity of the first stage of Erbaa Wastewater Treatment Plant (Q-design) is 11,640 m3/day and according to the design projection, it is foreseen that this capacity will be sufficient until 2022 and the capacity will be increased with the construction of the second stage after 2022. The project was submitted to the Ministry of Environment and Urbanization on 16.05.2016 and approved by the Ministry on 28.06.2016. As of 2023, 96% of the existing wastewater treatment plant with a capacity of 11.640 m3/day is used. Erbaa Municipality prepared the Existing Wastewater Treatment Plant Capacity Increase Feasibility Report in June 2021 and updated it in February 2024. In November 2022, Erbaa (Tokat) Municipality Wastewater Treatment Plant Study Report was prepared. Related reports are given in Annex 1. In the area where the plant is located, the necessary expansion area has been left for two additional aeration and two settling tanks to be built for the second stage.

The populations used and calculated in the design of the existing Erbaa Wastewater Treatment Plant are given in Table 2 below.

Table 2 Population to be Served by WWTP

| Year | Population (person) |
| --- | --- |
| 2007 | 56,076 |
| 2022 | 87,365 |

Source: Erbaa (Tokat) Sewerage Final Project, Project Report, 2014

The flow rates predicted in the design report are given in Table 3 below.

Table 3 Design Flowrates

| Flowrates | Hourly Wastewater Flowrates (m3/hr) |  |
| --- | --- | --- |
|  | 2007 | 2022 |
| Minimum (Q37) | 233 | 326 |
| Daily Average (Q24) | 335 | 485 |
| Project Flowrate (Q16) | 499 | 741 |
| Maximum (Q12) | 653 | 981 |

Source: Erbaa (Tokat) Sewerage Final Project, Project Report, 2014

Satellite images of the existing facility are given in the figure below.

Figure 6 Erbaa Municipality Existing Wastewater Treatment Plant

Source: https://www.Erbaa Belediyesi.gov.tr

The general layout of the existing facility is given in the figure below.

Figure 7 Erbaa Municipality Existing Wastewater Treatment Plant General Layout Plan

Source: https://www.Erbaa Belediyesi.gov.tr

The flow diagram of the existing plant is given in the figure below.

Figure 8 Erbaa Municipality Existing Wastewater Treatment Plant Flow Diagram

Source: https://www.Erbaa Belediyesi.gov.tr

Erbaa Municipality has carried out a series of mechanical equipment additions and renewals by making improvements in the wastewater treatment plant in certain periods. The existing wastewater treatment plant of Erbaa Municipality consists of the following units;

- Physical Treatment

  - Mechanical Coarse Screen

  - Automatic Fine Screen

  - Pumping Chamber

  - Grit and Grease Removal Unit

- Biological Treatment

  - Anaerobic Tanks (Bio-phosphorus Tanks)

  - Aeration Tanks

  - Sedimentation Tank Distribution Chamber

  - Sedimentation Tank

  - Return Sludge Tank

- Sludge Dewatering (Decanter)

- Buildings

  - Administrative Building

  - Blower Building

  - Sludge Dewatering and Workshop Building

  - Guard House

  - Transformer Building

The Wastewater Treatment Plant is currently operated by Erbaa Municipality to meet the discharge criteria of the Urban Wastewater Treatment Regulation. The treatment plant is designed as an extended aeration activated sludge system. Carbon removal is provided in the plant, but nitrogen and phosphorus treatment are discharged at a quality close to the limit values.

The current capacity utilization of the existing WWTP has reached 96% and it seems possible to increase the capacity by 5% to the extent permitted by the operating parameters. However, with the capacity increase in the near future, a flexible (margin) process design is required to operate at more reasonable operating parameter ranges during operation.

The average measurement results of the inlet parameters for 2007 and 2022 were provided by the Erbaa Municipality wastewater treatment plant operation chief. The measurement results for the parameters which are the basis of the design are given in Table 4 below.

Table 4 Inlet Pollutant Parameters

| Parameter | 2007 | 2022 | Discharge Limits |
| --- | --- | --- | --- |
| COD (mg/l) | 478 | 514 | 100 |
| BOD (mg/l) | 220 | 237 | 25 |
| TSS (mg/l) | 277 | 298 | 30 |
| TKN (mg/l) | 47 | 51 | 15 |
| TP (mg/l) | 7 | 8 | 2 |

Source: Erbaa (Tokat) Sewerage Final Project, Project Report, 2014

### Receiving Body

The treated wastewater from the wastewater treatment plant is discharged into Kelkit Creek.

The wastewater treatment plant is designed and constructed to comply with the discharge effluent criteria set out in EU Urban Wastewater Treatment Directive (91/271/EEC), Turkish Water Pollution Control Regulation (Official Gazette, dated 31 December 2004 and no 25687), Urban Wastewater Treatment Regulation (Official Gazette dated 8 January 2006 and no 26047) and the “Urban Wastewater Treatment Regulation, Communiqué  on Sensitive and Less Sensitive Water Areas” (Official Gazette, dated 27 June 2009 and no 27271) ,

## Surveys

### Topographical surveys

The Contractor shall familiarise himself with the site conditions before submitting his tender. This survey is given only for information purpose. The Contractor shall conduct a new topographical survey for his design works.

### Geotechnical investigations

A geological and geotechnical survey carried out for the existing WWTP report is provided in Folder 1.

As the capacity increase of the treatment plant will take place in the area where the existing treatment plant is located, the geological survey values of the existing treatment plant are used.  It is known that liquefaction occurs in the area in question and ground improvement works were carried out during the construction of the existing treatment plant. As the area where additional investments will be made is within the same area, ground improvement works will be required. The tender documents state that geotechnical investigations will be carried out during construction.

The geotechnical evaluations are subject to change according to the plant structures and layout in the actual plant design. The survey is given only for information purpose. The Contractor shall conduct new soil investigations according to the locations, dimensions, foundation depths and weights of the structures in his detailed design. The Contractor surveys should meet the below requirement;

- At least one borehole for each main process units,

- In new survey, the depth of borehole shall be at least 15 m if it shall not meet with the main rock. In cases where the number of SPT pulses "N> = 30" on cohesive soils, and the number of SPT pulses "N> = 50" on cohesionless soils, the drilling shall be end in 3m. If it shall meet with the main rock, it will proceed in 3 m. If the main rock shows a cracked feature, it will proceed in 5 m.

- For samples and experiments should be performed according to Ilbank Specification.

The methodology of excavation and the geotechnical precautions are subject to the Project Manager’s approval. The Contractor shall prepare the excavation plans and sections and geotechnical design in accordance with the method statement approved by  the Project Manager.

Erbaa (Tokat) Sewerage Final Project Project Report

Existing Wastewater Treatment Plant Capacity Increase Feasibility Report, June 2021, Erbaa Municipality

Erbaa (Tokat) Municipality Wastewater Treatment Plant Study Report, November 2022, İsmail Altay, Harun Sungur